# Generation of Audio signals using a Raspberry Pi Pico

Jack D. Morrison

ID: 190009927

AC40001 Honours Project

BSc (Hons) Applied Computing

University of Dundee, 2023

Supervisor: Dr. Andrew Cobley

Abstract –

The aim of this project is to develop a Voltage controlled Oscillator using a Raspberry Pi Pico and integrate it into a euro rack synthesiser format. The product uses pulse width modulation to create an analogue oscillating voltage using a transformation circuit. This required investigating 2 methodologies for determining the duty cycle of the pulse width modulation circuit, using wave tables to determine the level or calculating the duty cycle at runtime. Further development went into both methods, developing different waveforms, with emphasis on the wave table solution that made its way into the final product. The Product was developed so that Oscillating frequency was determined by a Korg SQ-1 sequencer and integrated within a euro rack format. The product was implemented with an additional Oscillating output and testing concluded that the frequencies that were produced were accurate within ±5% of the desired frequency. The waveforms were also produced accurately and the overall the project was successful.

# Introduction

The objective of this project is to develop a Digitally controlled oscillator capable of producing audio rate frequencies using a Raspberry Pi Pico. This report will detail the extent to which the inexpensive Raspberry Pi Pico was developed to become a voltage-controlled oscillator and how it was integrated into a euro rack synthesiser format. The outcomes of this project will be a final product that should be an inexpensive commercially viable voltage-controlled oscillator that can change the shape of the output wave form. The methods researched in this project could additionally be integrated into other commercially viable audio synthesis devices for example it could be integrated into a toy keyboard for kids to learn how to play piano or into a synthesiser pedal for electric guitar.

The Raspberry Pi Pico is a microcontroller which can generate digital outputs which means that the output voltage can only be on or off meaning that the only wave form it can produce is square wave. This poses a problem when generating an oscillating voltage with analogue waveforms such as triangle and sinusoidal. Hence a method of converting the digital output into analogue is required. This problem requires pulse width modulation to convert the digital output to analogue. Pulse width modulation is a method where analogue voltage size is determined by modulating the duration of a voltage pulse. The pulses then go through a transformation circuit containing a capacitor routed to ground which stores the voltage from the pulse for the duration of the pulse. The voltage from the capacitor will rise to the voltage of the voltage supply over time, the voltage across the capacitor after a t time period is given by the equation VC=VS(1-e(-t(RC)) where VS is the voltage from the Pico, e is the Euler irrational constant ≈ 2.7182, t is the time of the pulse, and RC is the resistance and capacitance of the circuit in ohms and Farads. Hence the resultant voltage from the capacitor that has a magnitude that corresponds to the duration of the pulse. This is used to generate a controlled analogue voltage which can be modulated by changing the duration of the pulses going into the capacitor.

The duration of pulses from the Pico is relative to the rate that pulses are repeated and is referred to as the duty cycle. The period of the repeated rate can then be used to determine the period of the output wave. The oscillating frequency can then be translated into the number of pulses that is required to reach the size of the period of the wave by the formula no.pulses=repeatedrate÷frequency. This is the start to generating a frequency since it is known how many pulses are in a single wave, the wave shape can be controlled by altering the duty cycle of the pulses from 0 to x number of pulses.

Two methods that will be investigated in this report which are using runtime calculations to find duty cycle and the other is generating the duty cycle first, then storing them in a table and then using table look up to find the duty cycle.

Both methods have their own use cases and limitations. The main benefit of runtime calculated duty cycle is that it is possible to alter the wave forms profile during runtime but is potentially limited to simplistic calculations since large complex calculations could take a large amount of time to compute and may reduce the frequency of the wave if it becomes backlogged with too many large calculations. On the other hand, the table look up method could allow for much more complex waveforms and a more stable frequency although this limits it in its ability to alter the wave profile. This project will investigate how both methods can be used to create unique sounds.

This report will also document how the frequency of the output is controlled by an input voltage. Which will document the development of the Pico to take an input from a Korg SQ-1 sequencer. A sequencer is a device that allows the user to construct sequences of musical notes by sending a control voltage to indicate a desired frequency. This means that the synthesiser will be able to take this voltage as an input and read the value so that it can produce the intended result. This project will document means of changing the frequency of the sound in an accurate way. One method that will be researched in this project is storing different frequencies in an array and looking up the frequency when required. This requires taking a base frequency and modulate the rate that the duty cycles are outputted to change the Frequency of the waveform. This allows for a wide range of frequencies that the system can produce and doesn’t require much memory usage.

# Background

## Programming the Raspberry Pi Pico in C

To research how PWM is integrated into the Pico in the C programming language, a section of the Book, ‘Programming the Raspberry Pi Pico in C’ By Harry Fairhead was researched. This book contains knowledge of the Pico PWM hardware which is explained in this section.

PWM can be integrated on any of the 30 GPIO pins which are divided into 8 slices. GPIO pins 0-15 correspond to slices 0-7 and GPIO pins 16-29 correspond with 0-6. Each slice has 2 output channels (A/B) which have 2 pins associated with it as shown in the table below based on Harry Fairhead’s explanation.

Figure 1. table showing the slice and channel allocations to the GPIO pins.

Each GPIO PIN has a slice associated with it, the PIN shares a slice with 1 other PIN and the hardware identifies each PIN with its slice number and the channel which could be either A or B depending on which PIN is required. These variable can be defined by using the pwm_gpio_to_slice_num(uint gpio) function which returns the slice number and pwm_gpio_to_channel(uint gpio) function returns 0 or 1 for channel A or B.

To enable pwm on a pin it needs to be initialise first by using the gpio_set_function(int GPIO PIN, GPIO_FUNC_PWM); so the hardware know that the function of the PIN is for PWM only.

To turn PWM off or on you can use the pwm_set_enabled(uint slice_num, bool enabled) function or if you have multiple signals that need turned off or on you can use pwm_set_mask_enabled(uint32_t mask). The mask represents the eight PWM slices as the first eight bits.

This also requires the target_link_libraries(hardware_pwm) inserted int the CMakeLists.txt file.

PWM generation in the pico work by iterating the counter at the clock frequency which means that the steps occur every 1clockfrequency seconds. You can increase the time between the steps by increasing the clock divider which is initially set to 1 which can be substituted into the formula so that TS=clockDividerFc where Ts is the time between steps and Fc is the clock frequency. The steps work with a 16-bit counter meaning that the can be stepped on a maximum of 216 times before wrapping around. The number of steps before it wraps can be configured to be any value between 1 and  216 by changing the “wrap” value in the PWM configuration. This higher the value you set the wrap to be the lower the output frequency of the PWM signal will become likewise the lower the value the higher the frequency but the lower the value the wrap the lower the granularity of the duty cycle. This is due because duty cycle is set to a value between 0 and the wrap. This is the case because the when the pico steps through the counter it outputs a the IOVDD voltage until it reaches the limit value.

In the documentation is lays out how to change the wrap size using the PWM_hardware library. The functions were pwm_set_wrap(uint slice_num, uint16_t wrap). In the normal mode it is explained that once the counter reaches the wrap value it starts from 0 again. On the other hand, there is a second mode that can be used called “phase-correct” mode which instead counts back down to 0 once the wrap is achieved then up to the count again. The normal mode mimics a sawtooth wave as you can see from the diagrams from the documentation:

Figure 2. Diagram of counter increasing in non-phase shift mode

And the Phase correct mode is a triangular wave:

Figure 3. Diagram of counter increasing in phase shift mode

This means that in phase shift mode the frequency is halved, but it also means that the pulses produced are active for double the time duration. As shown by the diagram the phase is corrected which means that pulses are centred on the 0 count of the counter. The documentation states that it can be used by implementing the pwm_set_phase_correct(uint slice_num, bool phase_correct) method from the hardware_pwm library.

The book also explains that to set the duty cycle of the pwm output you must set the channel’s level. There are 3 methods for doing this, first is using the pwm_set_channel_level(uint slice_num, uint chan, uint16_t level) which takes in the slice of the channel, the channel (A or B) and the level. Secondarily both channels can be set using the pwm_set_both_levels(uint slice_num, uint16_t level_a, uint16_t level_b) which has separate level parameters for channel a and b of a given slice. The third method is a helper function pwm_set_GPIO_level(uint GPIO, uint16_t level) which will set the level on the specified GPIO pin.

The Book also explains a couple of helpful equations that can be used to find the frequency of the PWM output based on the clock frequency (Fc) and the wrap. The equation looks like F=Fc(wrap+1) for non-phase correct mode hence the wrap to produce a frequency can be found by the equation wrap=Fc(F-1) . Fairhead also states an equation for finding the level of a duty cycle as level=wrap*duty, with duty being the duty cycle as a fraction.

He also specifies the same equations for phase correct mode as F=Fc2(wrap+1) and wrap=Fc(2F-1) to account for the decrease in frequency.

This can also be modified by the clock divider which divides the Fc value in the previous equations. The documentation has 2 methods for doing this which are pwm_set_clkdiv_int_frac(uint slice_num, uint8_t integer, uint8_t fraction) and helper function pwm_set_clkdiv(uint slice_num, float divider). Fairhead states in his book that “The divider is decomposed into an 8-bit integer and a 4-bit fractional part which specifies the fraction as fract/16.” Hence you can find the new clock frequency by dividing by the integer, then dividing by fract/16.

Research began by looking into PWM with the Raspberry pi Pico which led to finding the “Hardware design with RP2040” Datasheet which contains a circuit diagram for use with a PWM audio source see figure 1 in appendix. The circuit from the datasheet has 2 PWM outputs from the Raspberry Pico which go into a small logic buffer. Then each signal passes through a 220ohm resistor, then past a 100 nano farad resistor and then a 100-ohm resistor, both connected to ground. Each signal is then connected to a 47 micro farad capacitor and then passed a 1.8k ohm resistor connected to ground. The effect this would have on the pulses outputted from the Pico would be that the logic buffer would clean up the signal with an external 3.3V supply before going into the rest of the circuit. The rest of the circuit stores the pulses inside the capacitors and release them when the pulse ends meaning that the longer the pulse the higher the voltage released from the capacitors.

Another place of research was Robin Grosset’s pico-pwm-audio github repository which contains an example of PWM using table look up. Inside the repository Grosset includes a C program that generates a PWM output when compiled into a UF2 file using the raspberry Pi Pico C build configuration in VS code. The C program uses the Pico standard library along with the interrupt, PWM and sync library to accomplish this. The C file defines a PWM audio PIN (28) and includes a sample.h file which has a macro called WAV_DATA_LENGTH and an unsigned int array called WAV_DATA this is the table that is looked up in his method. It contains values ranging from 0 to 255 to represent the pulse widths used to replicate the sample audio signal. The code also overclocks the Raspberry Pico to 176000Mhz and initialises the pin with a function from the PWM library that sets up the PIN in PWM mode. It then creates an int value called audio_pin_slice which is set to the PWM slice which a value attached to the GPIO PIN which tells it when to fire. It then clears the PWM channel interrupt on the PIN and Enables a single PWM instance interrupt and sets the handler of the interrupt to a function defined before main () called pwm_interrupt_handler which checks its wave position is less than the wav data length multiplied by 2^3, utilising bit shifting to increase efficiency. This is because each value is repeated 8 cycles. It then sets the value of the pulse width and increases the wav_position. Also includes an else to reset the wav_position to start.

In the final part of the main it sets a pwm_config to the default config provided by the hardware/pwm.h file and sets the clock divide to 8.0 which adjusts the rate of events seen on the PWM PIN before passing them on to the PWM counter. This value changes the rate at which events fire in other words. It then sets the wrap to 250. The wrap sets the highest value the counter can reach before turning to 0. It then initialises the pin with the configuration and start parameter set to true. After initialisation the GPIO 28 PIN’s level is set to 0 and starts an infinite loop that waits for interrupts to wake up the core and execute the handler function.

The grosset repository contains a great approach to PWM and it is certainly one way of doing it and wasn’t particularly well explained so a lot of the knowledge gained was using the Raspberry pi pico library documentation to find out what the methods were doing but was a bit vague in explaining the actual functioning of the methods.

Another helpful repository that was researched was Jake Rosoman’s pico-button.c and pico-gpio-interrupt.c both used under MIT license. These C programs provides helper structs to initialise multiple button inputs and handlers for interrupts.

The pico-gpio-interrupt.c program first defines a pointer to a handler function type with a pointer to argument as a parameter. Then it defines a closure_t struct which contains 2 member a pointer called argument and a handler function member called fn using the pointer-to-function type. Then it defines a 28-item array of closure_t structs for each gpio which is set initially null. Then it defines a handle_interupt function that is called when a interrupt is created hence is contains uint gpio and uint32_t events parameter. In this handler the gpio pin is used to find the closure_t item from the array defined above and then uses then calls the fn function of the struct using the structs argument parameter. Finally, a listen function is created which takes in a pin number, condition (which is the event that causes an interrupt), a fn pointer-to-function handler type, and a pointer to an argument. This function initialises the gpio pin and pulls up the pin, then enables interrupts with callback for the pin using the condition parameter to set the event and set handle_interupt function as the callback function. Then a closure_t object is created and then fn and argument members are set using the parameters of the function. Then the closure_t array is updated so that the gpio pin number’s item in the array is set to a pointer to the newly created closure_t structure.

The button.c program includes the gpio-interrupt.c program and uses the listen function to initialise the button pin. The program first defines a struct called button_t which has 3 members, a uint8_t member for the button pin, a bool member for the state of the button which is used to determine if the state of a button has changed in the handle button alarm and a pointer to a function with parameter of a button_t structure. Next the program defines a function that called handle_button_alarm which takes in a void pointer p which is then converted into a button_t pointer which calls the onchange function if the button state has changed. This is used to add an alarm so that the button  onchange function is delayed 200 micro second. Then a create_button function is added that returns a pointer to a newly allocated button_t structure. The function takes the pin number and a pointer to the onchange handler function as parameters. Then allocates a button_t struct in memory. Then it uses the listen() function to initialise the button pin with the RISE and FALL events as well as the handle_button_interrupt function, which is set to the call back function. It also sets the argument parameter as the newly allocated button_t structure so that the handler_button_interupt function uses it as the parameter. Then the struct members of the structure are initialised with the parameters. In the button.c repository it contains a example.c program which shows how this can then be implemented into a C program, by including Button.c, defining a onchange function and initialising a button_t struct pointer with the create_button function with the pin number and onchange function as parameters.

# Specification

The final product of this project must use the Raspberry Pi Pico to produce an oscillating voltage using PWM and the frequency of the output must be externally controlled by a controlled voltage. The product must be integrated with a Korg SQ-1 pitch sequencer which requires that it uses the Korg SQ-1’s CV A to control the frequency and could use the GATE-OUT to determine when the frequency is to be read. The product must use the 1V output option from the SQ-1 to modulate the frequency in a 1-octave range. The product should be able to use the 2V output option from the SQ-1 to modulate the frequency in a 2-octave range. The product could be able to use the 5V output option from the SQ-1 to modulate the frequency in a 5-octave range. The product must be integrated into a Euro Rack synthesiser format. This may be integrated using 2 ADSR modules attenuating the volume of a voltage controlled low pass filter and amplifier modules. The product should be able to output different waveforms which could be Sine waves, square waves, triangle waves and parabola. The product should also have button inputs so that the different Wave Form options can be selected. Additionally, the product could combine waveforms in runtime this could be done with different notes to make chords or add harmonics to simulate plucked notes. The product could additionally implement vibrato functionality that modulates the frequency up and down a semitone. The product could integrate another oscillating voltage using PWM, the frequency of which could be determined by the CV-B output from the Korg SQ-1. This could also lead to a secondary euro rack integration with a ADSR and amplifier modules. The Secondary could have a secondary waveform selection allowing for independent control of wave form for each output.

# Design

From the outset of the project the Coding language that was chosen was C. This was chosen because C is much faster than the other option, micro-python because micro-python is converted into C then compiled, the result is much slower than compiling the C code into the UF2 binary.

The SDK used to compile the code was the Raspberry Pi Pico C/C++ SDK for Linux operating system. An Ubuntu VM was used for the SDK because the windows version seemed to have a lot less support and the installation process was a lot more streamlined. This SDK was used due to its integration of the pico standard libraries and its CMake compiler’s ability to compile C programs into UF2 files that are used on the pico.

PWM the circuit which was inspired by the diagram from the “Hardware design with RP2040” Datasheet by Raspberry Pi.

A script was required to create a header file based on some configuration settings that are inputted at runtime. Python was the chosen language for the script due to the student’s technical proficiency with using python and python suited the mathematically technical scripting involved with generating the arrays that were in the header file.

# Implementation and Testing

## Overview

The first step of implementation was creating a working circuit that could take a PWM output from the Raspberry pi Pico and output a oscillating analogue voltage. Next was to develop methods of generating a consistent and accurate frequency so that an output frequency could be determined programmatically ideally so that an integer value in the Program corresponds to the value in Hz out from the circuit.

The next stage of implementation to reach the final design specification is to take an analogue voltage and convert it into a digital value and use it to determine the frequency output.

Once the frequency can be altered by an input voltage the next specification to address is the alternate waveforms these consist of sinusoidal, triangle, sawtooth, reverse sawtooth, and parabolic waves. This required a method for generating wavetables, and a method for alternating between them.

With the wavetable solution addressed the next step is to find a runtime solution to produce these waves, and then introduce a controlled voltage input to change the frequency. The wave profiles of some waves can then be modified to take in another controlled voltage to alter the duty cycle of a square wave and the angle of a triangular wave.

The next stage is integrating sinusoidal harmonics into the waveforms at runtime. This also means implementing another method to add or remove harmonics from the wave.

After the waveforms have been fully explored the next step in implementation is taking the results and integrating it into a euro rack synthesiser.

This first involved using a Korg SQ1 sequencer to alter the voltage. This step also involves modifying the prototype to take a voltage per octave format and accepting a 5V signal that allows the Pico to span 5 Octaves and a gate which tells the Pico when to play notes. The next step is then routing the signal through a voltage-controlled filter and then through a voltage-controlled amplifier. These also contain an effects loop which allows for modulation of the attack, decay, sustain and release of the Wave form produced by the Pico.

The next developmental goal to achieve was implementing a method of generating vibrato where the frequency alternates up and down by half step intervals. This meaning that the frequency goes up a semitone then down to the frequency and then down a semitone then back to the frequency and then repeats.

Once the pico is fully integrated within the circuit the final stage is to have 2 PWM signals produced by the Pico. This involved taking integrating another ADC input and another gate input as well as another PWM circuit. This was also integrated within the waveforms were integrated so that each signal’s waveforms can be altered independently.

## PWM Circuit

The first step of implementation was creating a circuit that can take a PWM output from the Pico and transform it into an oscillating voltage. The circuit for this first prototype looked very similar to the RGrosset implementation as shown by the diagram.

This implementation used the same GPIO 28 as grosset but instead of using an Audio Jack the output went straight into an oscilloscope. Another slight change was the use of a 330Ω, 470Ω and 1KΩ resistor in parallel instead of a dedicated 1.8KΩ resistor.

This prototype was first tested using RGrosset’s pico-pwm-audio code from his GitHub and was compiled on an Ubuntu virtual machine with the Raspberry Pi Pico C/C++ SDK for VS code installed. The compiled UF2 file was then transferred onto the Pico for testing. The results of the first prototype showed several different frequencies which indicated that the circuit had been implemented correctly.

## Accurate Frequency

After the circuit was established, the next step was to generate a single frequency which is generated so that it produces a specific frequency and tested to ensure the frequency is generated with the expected frequency. This was achieved by first forking the code developed by RGrosset and removing the the sample.h, ring.h, that’s_cool.h, circuit diagram.png and the Raspberry Pi Pico PWM Wave File Converter.ipynb. The pico-pwm-audio.c file was also updated to use a new .h file created by a python program.

The python program was Implemented to take in a frequency and sample rate as an input and then generate an array that is written to the new.h file and used to modify the duty cycle of the pwm output.

The first iteration of the python program generates Pulse widths for a single sine wave. This was achieved by defining a variable for the period of the wave equal to 1/Frequency then create a multiplier variable = 2 times pi over the period of the wave, which is referred to as B in the program, this variable, B, is used to find the duty cycle to equal the amplitude at each point along waves cycle. When the wave is reproduced, pulses are generated at a certain rate which depends on the clock frequency and clock divider hence it is also of use to find the interval size(1/samplerate) which is the size between 8 calls of the handler function in RGrosset’s C program which are called at a rate of clock frequency/clock divider. The interval size was used to iterate until it has reached the end of one cycle of the wave.  At each iteration the duty cycle at the point is found by using a sine function of the variable B multiplied by the size of the intervals iterated through. With the duty cycle at that point found we need to create a value that the pico uses to determine the duty cycle. The sine wave function alternates between -1 and 1 but this is no use for PWM because the value needs to be a positive integer from 0 to the size of the PWM wrap. Which requires addition of one to the value to make it positive then multiplied by the size of the PWM wrap to justify it for the program. It then writes the data as an C array to a header file which is included in the C program.

The RGrosset Program was then modified to use the Python generated array by includin. It was then compiled and tested through the oscilloscope by checking that the period of the waves are constant and of equal length to the period defined in the python program.

## Voltage Controlled Frequency

The next step in development was the use of a controlled voltage to determine the frequency of the PWM output. This involved implementing 3 parts, creating a controlled voltage to alter the frequency, converting the analogue voltage input into a digital form, and changing the frequency of the signal with digital value.

### Controlled Voltage

The controlled voltage used to change the frequency of the output was implemented by using a 10kΩ potentiometer and the 36th PIN (3V3(OUT)). The Vcc pin of the potentiometer was connected to the 3V3 OUT PIN and the output pin connected to the GPIO 26 PIN with the ground pin of the potentiometer connected to a ground pin on the Pico as shown by the diagram below.

### Analogue to digital conversion

The analogue to digital conversion was implemented using the “hardware/adc.h” library. This was used to initialise the GPIO 26 pin for use in adc using the adc_gpio_init() method in the main function. Then using adc_select(0) to select the GPIO 26 PIN. We then set a constant called conversion factor in the global scope which converts the adc_read() value to an integer representation of the voltage.

Then, inside the handler function we define the adc_value to equal the adc_read() multiplied  by the conversion factor each time the wav_position has equalled the size of the waves array length.

### Altering the frequency

The adc_value variable is then used to alter the frequency of the output. This was done by altering the clock divider which means the core frequency is divided into parts. The original clock divider used was 2.0 meaning the 176Mhz core clock frequency is divided by 2 for creating an effective frequency of 88Mhz which if we use the formula F=Fc/(wrap+1) knowing that we have a wrap of 250. We get an output frequency of 352,000Hz which is iterated through 8 times to iterate through a position in the wave so effectively we have an output frequency of 44,000Hz. If we increase or decrease the Fc by changing the clock speed we can increase or decrease the rate at which the values are outputted from the Pico. Which means that if we increase the clock divider we can increase the frequency likewise if we decrease the clock divider we decrease the output frequency. The next step is to take create a function that returns the clock divider necessary to generate a voltage. This was implemented by taking the new frequency as a float parameter and using 2 globally defined constants called WAV_FREQUENCY which is the frequency of the output with the original clock divider and clkDiv which is the original clock divider. The function divides the WAV_FREQUENCY constant by the new frequency to create a coefficient of the difference between the frequencies, then multiplies the coefficient by clkDiv to give the clock divider needed to achieve the new frequency. This value is then returned out of the function. This function was implemented to alter the frequency by first multiplying the WAV_FREQUENCY by the adc_value to create a newFrequency so that the frequency will increase as the potentiometer is turned. Then I used the function to find the new clock divider and then set the new clock divider using the set_pwm_clkdiv(int slice, float clkDiv) method with use of the slice variable used in the initialisation of the PWM pin, which put into the global scope so it could be accessed in the handler function, and the new clock divider.

### Testing

This was then tested using an oscilloscope by turning the potentiometer knob around and testing the if the frequency was altered. Then I used the oscilloscope to determine the voltage from the potentiometer with a slight turn of the knob and then calculating the what the WAV_FREQUENCY multiplied by the voltage would give. Then I attached the oscilloscope back onto the PWM circuit and read the frequency of the output to see if it corresponded with the value.

## Alternate Waveforms

### Overview

Like the sinusoidal waveform the other input arrays for the alternate waveforms were implemented using a python program to generate the values then writing them to the header file. The program was also altered so that all the alternate wave forms were generated from methods of a class named ‘wave’ which has fields of frequency, amplitude, sample rate, period, B, interval, and wrap. These values were used in the methods which return a string of the input array used to reproduce the wavicles. All the fields are set in the constructor which takes in frequency, amplitude, sample rate, and wrap as parameters and calculates the other fields using them as described in the accurate frequency section.

### Square Wave

The square wave was implemented using a while loop that iterates through using a variable called x which increase by 1 interval each loop until it has reached half of the period length + one interval.  Before the loop a variable called vals is set to equal the wrap field but converted into a string. Inside the while loop, the ‘vals’ variable is concatenated with ‘, ’ then concatenated with the wrap field converted to a string. Another while loop was then implemented which again iterates through using x which increases by 1 interval each cycle but until it equals the period field + 1 interval. The while loop similarly concatenates ‘, ’ to the ‘vals’ variable but instead concatenates “0” after.  Then the vals variable is returned from the method.

### Triangle Wave

Similarly, the triangle wave implementation was implemented by looping through in different fractions of the wave. The method starts with defining the tan_theta variable which is used to find the duty cycle at each point along the period of the waveform. This is set to 4 times the amplitude field divided by the period field because a triangular wave in this implantation it is split into 4 triangles with a height of 1 amplitude with a width of ¼ of the period of the wave. The quarter wave implementation was used so that the wave starts with a duty cycle of 50% just like the sinusoidal waveform. A variable named x is used to iterate through the wave and is used set the first value which is x multiplied by tan_theta and then adding the amplitude field and dividing it by 2 times the amplitude to give the duty cycle which is set to variable v. This was then used to initialise the vals string by taking the v variable and multiplying it by the wrap to get the level at runtime. Then a while loop is used to iterate through the first quarter of the waves period using x where it adds the values to the string using the same method previously mentioned. Then it iterated through the next 2 quarters of the period again using x. In this loop x times tan_theta is used to subtract from 3 times the amplitude and divided by 2 times the amplitude to give the duty cycle then multiplied by the wrap to give the level at runtime.

The final while loop iterates through the last quarter of the period again using x. Where it finds the value of x times theta and justifies it to give the duty cycle by subtracting three amplitudes then dividing it by 2 amplitudes. Then it returns the vals string which are the values found by iterating through the wave concatenated as strings and separated by commas.

### Sawtooth Wave

The sawtooth wave was implemented much in the same way as the triangle wave, instead of using the 4 subsections instead I used just 2 sections this meant that the tan_theta value changed to 2 times the amplitude divided by the period field. Similarly, to the last wave it iterates variable x from 0 to half of the period + interval. We then multiply the value x by the tan theta to get the value and then concatenate the string with the value plus amplitude divided by 2 amplitudes, multiplied by the wrap and turned into a string. Then I added another while loop to iterate the x variable further until it reaches the period plus the interval in another while loop that finds the value but instead concatenates the value minus the amplitude divided by 2 times the amplitude. This means that the value is justified to the lowest duty cycle. Then it returns the string of the values separated by commas.

#### Reverse Sawtooth Wave

This was created by copying the previous function but instead the value was calculated by subtracting the x times tan_theta value from the amplitude when its concatenated in the first loop and in the second subtracting it from 3 times the amplitude when concatenating it.

### Parabolic Wave

The method for generating the parabolic wave input array was implemented again by iterating through using the x value from 0 to the period plus one interval. The value at each iteration was found by squaring x minus half of the period divided by half of the period. This can be used because as x rises toward half of the period the fraction of x-0.5T0.5T.increases towards 0 from -1 and since it is then squared the value decreases exponentially until zero. Then from 0 to 1 it increases exponentially. The value is then multiplied by the wrap and concatenated to the vals string and returned from the method.

### Harmonic waveforms

The harmonic waveforms were implemented much later in the program’s development. The harmonic waveforms were created by copying the sine wave method but instead iterating using a variable called h from 1 to 12, the h variable was used in creating a new B variable that multiplies the frequency field by h+1 and multiplying that by 2 times pi. Then I use that variable to find the values of the harmonic’s sine wave input array and concatenates it to a string then adds the string to an array of input array strings and then returns the array with the values from each of the harmonics.

### Writing to the header

The python file was then modified to create a new wave class using variable that were found using pythons ‘input()’ method that allows easy changes to the frequency and sample rate which is useful when testing the project. These values were then used to initialise the wave object which is then used to define variables for each waveform by calling the specific generate waveform method. Python’s open() method was then used to open the header file which required assigning a variable f as the file object and called the write() method from it to write to the header. A first thing to write to the header file was 2 macros the WAV_FREQUENCY macro used to alter the clock divider and the WAV_DATA_LENGTH. This was done by writing “#define WAV_FREQUENCY” then converting the int value from the input method into a string and writing that to the file then writing ‘\n’ to the file to break the line. The WAV_DATA_LENGTH was done in the same way except it found the value of the macro by dividing the sample rate by the frequency. Then I wrote a constant of the new clock divider found by dividing 88,000 by the new sample rate 88000 was found by taking the clock frequency and dividing it by the wrap and number of times it repeats a value in the pwm handler (8). The waveform input array strings are then written to the header file by writing "uint8_t *wave name*_WAV_DATA[] = {\n" to the file then writing the string input array variable to the file followed by “};\n”. The harmonics shared the similar format except that it iterates through the array outputted by the wave object in the format of "uint8_t HARMONIC*number*_WAV_DATA[] = {\n".

### Testing

To test the wave forms a button was connected to GPIO PIN 10 and the 3V3OUT PIN meaning that when the button is pressed the GPIO 10 PIN gets a HIGH reading. The PIN was set up in the C program using a macro called WAVEBUTTON set to 10 and in the main function gpio_init() was used to initailise the pin and gpio_set_dir(WAVEBUTTON,GPIO_IN) was used to set the direction. The PIN was implemented to use a handler function which was enabled with  gpio_set_irq_enabled(WAVEBUTTON,GPIO_IRQ_EDGE_RISE ,true). This also sets the interrupt to be initialise when the voltage across the pin is raised up. The handler function was then assigned to the pin using gpio_add_raw_irq_handler_masked(( 0x01 << WAVEBUTTON),&rawHandler1) function which take in a bit mask of the integer value of the pin which is converted in this case by bitshifting the 0x01 value up by the magnitude of the WAVEBUTTON macro. Then we link to the memory address of the rawhandler.

The raw handler first checks to the event mask of the interrupt are from the WAVEBUTTON pin and is from a rise event. If so then it acknowledges the interrupt and iterates an integer variable declared in the global scope called button between 0 and 5. This variable is then used in the pwm interrupt handler function in a switch case that has 6 cases for each value 0 to 5, each corresponding to a waveform, the switch case is situated where the pwm_set_gpio_level is called. In each case of the switch statement the pwm_set_gpio level is set to equal the corresponding input array value at the given wave position.

This means that when the button is pressed it changes the input array and subsequently, the wave form that is produced. This was then tested using an oscilloscope to ensure that the frequency of waves stayed constant, and that the amplitude of the oscillating voltage stayed constant independent of the waveform. The waveforms also had to conform to the specific shape of the wave form.

### Button.c Integration

The final solution was implemented using Jake Rosoman’s pico-button.c and gpio-interrupt.c code from jkroso’s github repositories. To implement his methods, the code from these files were integrated into a single c program named button.c which was included in the pico-pwm-audio.c program. The code from the raw handler was removed and the code from the main which initialised the button was removed. The circuit was also modified to include 9 buttons with one side of the buttons connected to ground and the other side connected to GPIO pins. 5 buttons corresponded to the 5 waveform, sine, square, triangle, sawtooth/reverse sawtooth and parabola and the other 4 were to add and subtract harmonics (even and odd separately). To integrate them in the C program macros were defined for each of the GPIO pins numbers that the buttons were connected to and a void function was added that takes a button_t struct pointer as a parameter. Then a local button_t object was then created with the use of this pointer. The local button object was then used in a switch statement to determine the pin number using its pin member. 9 cases were added to the switch for each of the button’s macros and the buttonNum for the five waveforms were assigned under the corresponding case. For the even and odd harmonic’s up and down buttons 2 new global integer variables were added called evenHarmonics and oddHarmonics which contain the number of harmonics to be added to the waveform at runtime. This meant that inside the cases for the harmonic even up the evenHarmonics integer was increased by one until it reaches 6 and for the harmonics down the evenHarmonics variable was decreased until 0. Similar logic was used for oddHarmonics but only iterates to 5 harmonics.

## Runtime solutions

### Overview

The runtime solution was implemented using a wavelength variable of a frequency which isn’t the distance wavelength but instead is the number of wraps cycles needed to achieve a frequency. This can be calculated as the clock frequency divided by the wrap size, the frequency, and the clock divider. This variable is the same as the array length of the input array method. In the implementation of the runtime solution this was divided a further 8 times because each value was repeated 8 times in the handler function. This was then used in runtime as an analogue to the period of the wave.  Hence an integer variable named wav_position was created to iterate from 0 to the ‘wavelength’ with each call of the PWM handler function and resets once the variable has exceeded the ‘wavelength’. This was created so that the duty cycle at a given point in a waves cycle could be calculated. Different methods were then implemented for each waveform to calculate the duty cycle using this variable.

### Square wave

The square wave duty cycles were implemented by checking if the wav_position variable was less than half of the “wavelength” and if true output a 100% duty cycle by setting the GPIO level to the size of the wrap. Else it will output a 0% duty cycle by setting the GPIO level to 0.

### Triangle wave

The Triangle wave was implemented by splitting it into 2 halves, the “up” half where the duty cycle increase from 0 to 100% in the span of half the wavelength and the down section that goes from 100% to 0. This meant the duty cycle was calculated taking the wav_position and dividing it by half of the “wavelength” then multiplying by the wrap to find the level to generate the required duty cycle for the up part. The down part was then implemented by first finding the fraction that the wav_position is between half the wavelength and the wavelength. This involved subtracting the wav_postion by half the wavelength and then dividing by half the wavelength. That value was then used to subtract 1 to give the invert the direction then multiplying by the wrap to give the level needed to produce the desired duty cycle.

### Porabola wave

The parabola waveform drops at an decreasing rate until the halfway point then rises at an increasing. This means that the duty cycle of the wave drops from 255 increasingly slow rate as wav_position increases at the same rate it decreased.

This method was implemented by first squaring the wav_position subtracted by the half of the wavelength then dividing by half of the wavelength. This was implemented in this way so that as the wav_position increases towards the halfway point it decreases in duty cycle gradient because the fraction becomes closer to 0 at an exponentially smaller rate and then the fraction becomes negative and continues to decrease until its -1 which is exponentially rising until the end of the loop. The value was then multiplied it by the wrap to give the level for the GPIO.

### Wave Shape control

The wave shape of the square and triangle waves was altered using the potentiometer input from the frequency alteration implementation. This meant that instead of taking the adc_value in the C program and using it to change the clockdivider it was instead used to determine the point along the period where the calculations change. In the square wave this meant taking the wavelength and multiplying it by the adc value to find the value of the pulse length which is the length that the square wave is set to 100% duty cycle. Then setting a limit so that the pulse length doesn’t exceed the wavelength. This meant that where the adc_read() is called that the value modifies a global variable named “pulselength” to equal the wavelength variable times the adc_read value multiplied by the conversion factor. This variable was then implemented into the handler function so that it checks if the wav_position is less than the pulse length then set level to equal the wrap.

## Korg SQ-1 sequencer integration

To integrate the Korg SQ-1 with the Raspberry Pi Pico it required reading the output voltage from the Korg SQ-1’s CV A output and using it to set the frequency of the output. It also required reading the OUT-GATE output and using it to determine when to update the frequency and when to play the notes when it is not connected to the ADSR.

This also required a integrating a different way of handling frequencies because the voltage from the SQ-1 didn’t translate to accurate notes when multiplied by the base frequency. This is because the SQ-1 uses the euro rack 1 volt/octave standard which means that every 1/12th of a volt corresponds to a semitone. Since the musical frequencies increasing logarithmically and this increases linearly there are some differences. This meant that the voltage had to be translated into which musical frequency it corresponds to. The next stage was implementing a tuning method to derive the different notes. First one to be implemented was just tuning.

### Just Tuning

Just Tuning was implemented by adding a method for generating semitone frequencies to the python file. This method was implemented in below the wave table generation. The method iterates through an integer value from 0 to 11, finding the value at each integer. It finds the frequency by multiplying the pitch ratio by the perfect first. The pitch ratios are as follows: m2(16:15), M2(9:8), m3(6:5), M3(5:4), P4(4:3), TT (45:32), P5(3:2), m6(8:5), M6(5:3), m7(7:4) and M7(15:8). It starts by finding the lowest octave’s frequencies which is one quarter the size of the base frequency. Then we find the frequencies for the following 5 octaves and write them to the file as constant integer array.

### 2 Volt Solution

The Korg SQ-1 has 3 volt/octave modes, 1V,2V and 5V with 1V being across 1 octave, 2 V being across 2 octaves and 5V having 5 Octaves. The first one to implement was the 2V solution which was implemented using analogue to digital conversion. The Pico took the input from a mono audio jack, that was connected to GPIO pin 26 and ground. The same methods as before were used for analogue to digital conversion to convert the voltage across the GPIO 26 pin into an integer representation. The adc_value was read in the handler function after the wav_position had been reset.  The frequency representation of the voltage was then found, still inside the handler, using an incredibly arduous if statement tree that checked if the value was greater or less than 1 if so then check if its greater than 0.5 then if its greater than 0.25, then 0.125, then 0.0675, then corresponding else statements to differentiate between each 0.0675 volts where it can then determine which frequency it corresponds to from the array. Once the frequency was found the change clock divider function was used to alter the frequency. This was then tested with the oscilloscope and frequency values were checked.

### OUT-GATE

The OUT-GATE was implemented into the Pico using a mono jack connected to a GPIO pin and ground. The pin was then initialised using gpio_init() and the direction was set using gpio_set_dir(). The GPIO pin was then pulled down to sets the pin low which for the gate means that the pin will shift to high when the gate is triggered. As with the button interrupts had to be enabled using gpio_set_irq_enabled so that when the high reading happens the handler function is called. Then a handler was associated with the pin using the gpio_add_raw_irq_handler_masked() function, using the same method as the button in the waveform solution to turn the macro into a bitmask. The same handler function was used but an additional if statement was added to check if the interrupt is from the gate and if the event that triggered it was a rise of voltage if so, then acknowledge the interrupt, take an ADC reading, convert it to the frequency and change the clock divider.  The ADC functionality was then removed from the PWM handler function. After it was tested by attaching the PWM output to a speaker to see if the frequency changed when the gate changed. Then a Global Boolean variable added to switch the signal off and on when the gate is off or on. This meant updating the raw handler for the gate to have an additional if statement to check if the interrupt is from the gate and if the event is a fall of the voltage, if so, then change the Play Variable to false. This also meant changing the Variable to true first Gate statement. Then an if statement was added that wraps around the logic in the PWM handler function that checks if the play variable is true. This was then tested through the speaker to ensure that output occurs when the gate is not active.  The final implementation:

### 5 Volt Solution

The 5V was implemented by first altering the input method, previously the CV-A was read in from an input mono jack wired directly to the GPIO 26 pin. If the output from the SQ-1 was set to 5V this would mean that the GPIO 26 pin voltage would exceed its stated capacity, hence the voltage had to be reduced to 3V. This required modifying the circuit so that the ADC pin was connected in the middle of 2 resistors that are connected from the mono audio jack output to ground. The resistance of the 2 resistors was based on the equation VR2=(VIN(R2))/(R1+R2) where the R1was chosen to be 2.2kΩ hence the other resistor was found to be 3.3kΩ by substituting in values VIN as 5V and VR2 as 3V the following calculation were done. 3 = 5*R2/2.2k+R2, 3(2.2k+R2)=5R2, 6.6k=5R2-3R2, R2=3kΩ.

The next step was updating the frequency finding algorithm to determine the frequency as 3V contains 5 octaves. This meant that each 0.05V differentiated a different frequency. Hence if statement tree was expanded to differentiate between each 0.05V which was implemented and tested before a better solution was found. The better solution divided the adc_value by 3 to create a fraction that represents its relative size to the max voltage and then multiplied by 60 to find the position in the array of the frequency. This meant that the frequency could be found by assigning this value to the subscript of frequencyList array.

This 5V solution caused some issues because when the frequency got higher than 1 octave then decreased below 1.0f and the capacitor got overloaded with current. Hence the solution had to be modified to fix this. This was mitigated by ensuring that the base frequency divided by the highest frequency and multiplied by the base clock divider was positive. This required changing the place of the base frequency in the array so that the base frequency was in the 3rd/4th octave, the wrap was also decreased to help increase the clock divider, but it then wasn’t producing enough current to fill up the capacitors in the low frequencies. Eventually the clock frequency was increased to 216Mhz which allowed for slightly less high frequency squealing but there was still an issue with the low notes not producing enough current. Once that was corrected by increasing the wrap or reducing the Base frequency to a lower octave, the higher notes did not balance well.

Hence the C program was then modified to repeat values just 1 time. This meant also changing the clock divider calculation to account for the 8 times increase in frequency. This had performed better than the previous methods and allowed for an increased wrap while still achieving good high frequency fidelity. The lowest notes still didn’t have enough current though.

The C program was later modified so that the base frequencies Octave’s wrap values were repeated 16 times in the lowest octave, 8 times in the 2nd Octave, 4 times in the 3rd Octave, twice in the 4th Octave and only once in the 5th Octave. The change clock divider function was then updated to check which Octave it’s in a return the correct clock divider, adjusted for the number of repeated values it has.  The base frequency then became the perfect 1st in the 3rd Octave. The find clock divider function was then updated so that if the new frequency is in the 5th octave, then it multiplies the new clock divider by 4 since it is repeating 4times less than the base frequency. Then we include a few else if statements to justify the clock dividers hence the frequencies in the 4th octave are multiplied by 2, the frequencies in the 1st octave are divided by 4 and the frequencies in the 2nd octave and divided by to. Then an else was added for frequencies in the 3rd octave. This solution meant that the clock divider’s lowest value will be 8/15 times the clock divider for just tuning since the highest frequency in an octave is the Major 7th which is 15/8 the size of the perfect 1st and this method means that each perfect 1st of an octave will have the same clock divider. This is because the frequency of the output is reduced when the duty cycle input is repeated. The final solution achieves this using a val global variable to store the bit shift that is used to increase the data length and reduce the wave position so that values get repeated 2^val number of times. The change clock divider function was then updated the val variable when a frequency change is made.

## Vibrato

The vibrato effect has the frequency of the output modulating between a semitone down and a semi tone up. The solution developed for this project has the frequency updating after every wave and changes at a rate that means that from the lower semi tone to the upper semitone it takes roughly 1/3 of a second for each note. This was achieved by setting the rate to the upper semitone frequency subtracted by the lower and divided by the current frequency/3.

A button was added to the circuit in the same set up as the Wave shape button this also meant adding a case to the switch statement of the onchange function to change the vibrato variable from true to false and false to true.

Once the input method had been implemented the PWM handler function was then modified with to include an if statement when the wave cycle has completed and the wav_position is set back to 0. This was modified by adding an if statement that checks if the vibrato Boolean value is true and if so, then modify the frequency. The frequency was modified by creating a temporary global frequency variable called currentF which holds the current frequency be that from the semitone below to the semitone above.

The direction of the vibrato depends on the Vibup Boolean variable that if true means the vibrato increases in frequency at every loop to the upper semitone else it descends to the lower semitone.  When the vibup is true the Frequency is checked against the upper semitone frequency and if its less than then it increases by the vibchange variable and if its larger than the vibUp variable becomes false. When the vib up variable is false it then checks if the currentF variable is less than the lower semitone if its larger then, it decreases the frequency by the vibchange parameter else the vibUp parameter becomes true.

The upper semitone variable, lower semitone variable and the vibchange variable are all updated when the gate handler gets called and after the new frequency is found so that each value gets updated for the new frequency.

## Euro Rack integration

### ADSR

The ADSR stands for Attack, Decay, Sustain and Release and the way it works in the euro rack is that it takes in a Gate input which tells it how long to play a note for and returns a controlled voltage that is used to modulate the volume of the synthesiser output. In this euro rack configuration 2 ADSR modules were added to the rack with one modulating the volume of the LFO and another modulating the amplifier module. The ADSR modulates the volume in 4 sections, attack which is the length for the volume to go from silent to maximum volume, decay which goes from peak of the attack to the sustain level, sustain which is the level that is maintained when the gate is on, and release which is the length of time after the gate has ended. This meant that the A channel GATE-OUT from the sequencer had to be removed from the Pico and the put through one of the ADSR modules and the B channel GATE-OUT put through the other. This required altering the code so that the adc_read() functionality was put inside the PWM handler so that an the ADC pin’s voltage was read after every cycle through a wave. This solution caused some differentiating frequencies because when the voltage from the SQ-1 was not perfectly sustained throughout a note resulting in dips in frequency throughout the span of a note. This was the result from testing with a speaker and readings from an oscilloscope. Hence a new solution was made that duplicates the gate from the sequencer by adding an additional mono audio jack and wiring it to another mono audio jack. This meant that the Pico was then capable of using the previous GATE input code to read in the CV from the sequencer.

### Other modules

The eurorack setup includes 2 ADSR modules, 1 attenuates the amplifier and the other attenuates the low pass filter.

## Dual PWM implementation

### Overview

The second pwm output was set up on a separate slice and was implemented with a separate set of variables for the slice number, pin channel, wav_position, vibrato variables, buttonNum, even harmonics, odd harmonics and val(for bit shifting in pwm handler). These were implemented into the program by first adding an if statement using a bitwise and operator to check if a variable irq set to interrupt status mask is equal to 1 bit shifted up by the slice number that is checked. The original code was then copied and put in the else if part of the statement that check for the second slice number.

### Second GATE-OUT

To take in a second GATE, secondary gate monojack was added and connected to GPIO 21. In the C program a macro was defined for the pin input and set up in the same way as the first GATE pin. This also meant adding an if statement to the raw handler that checks for GATE2 then the functionality from the first was copied into it and the new global variables were interchanged.

### Second ADC

To achieve 2 ADC inputs another mono jack was added to the circuit with the same resistor configuration as in section 5.7.2. Then an additional macro was defined for the second ADC pin, this required changing the PWM pin to GPIO 18 and moving the original ADC pin to GPIO 28 and setting the second ADC to GPIO 27. The circuit was then updated to account for the changes. Then the Gate handler was updated so that it selects the correct ADC pin before the read for both GATEs so that ADC 2 is selected in gate 1 and ADC1 is selected for gate 2.

### Second PWM setup

The main was updated to include a secondary pwm initialisation for the new PWM pin. Which meant initialising the PWM config with clock divider and wrap then initialising the pin.

## Python script final version

The final version of the python was renamed to configuration.py and the header file name was changed to configuration.h. It was also adapted to contain logic to ask for base frequency, repeated rate, wrap and clock speed values as an input then checks to ensure that the values are valid. It tests these values by first checking if the clock speed is within the range of 125-220 Mhz and validates that the repeated rate, wrap and frequency are valid by ensuring the base clock divider is sufficiently to allow for the frequency to be modulated up to the highest value and down to the lowest value.

# Evaluation / Testing

The first implementations of the accurate frequency showed a disparity of the frequency output through the oscilloscope due to a misunderstanding of the clock divider and wrap function in the PWM output. The mistakes were corrected, and the frequency output was read to be roughly accurate to the intended frequency, but it was difficult to read an accurate frequency with the oscilloscope as it had an inaccuracy of ±5% Hz.

Testing of the accurate frequency showed that if the repeated rate of the PWM output was set to around 11kHz then visible plateaus throughout the wave form but when increased past 20kHz the effect was removed.

The results from the testing wave form produced with wave tables showed that the square wave slopes a bit off instead of cutting straight to zero due to the capacitor taking time to discharge. The triangle waveforms were accurate with clear lines with a little rounding at the peaks of the triangle. The sawtooth, like the square wave has a slope off after the peak of the wave but is much less prominent and drops faster than the square wave. The parabola wave looked as expected. The results from adding the harmonic waveforms at runtime looked as expected.

The Results from the runtime solutions were surprising since it the addition of the calculation didn’t seem to affect the wave shape or cause any delays in the output wave form even with the addition of the adc_read() after every wave. The wave shape of the wave forms was as expected and the changes to the pulse width parameter had the expected result. Even with the parabola waves no delay was observed which shows that the Pico may be capable of doing even larger calculation.

Write about how phase correct mode could have been implemented into the runtime solutions alongside removing the repeating values part.

# Description of the final product

The final product uses the Raspberry Pi Pico to produce an oscillating voltage using PWM with the frequency of the output externally controlled by the Korg SQ-1 pitch sequencer using the SQ-1’s CV A to control the frequency and uses the GATE-OUT to determine when the CV-A voltage is to be read. The product can use the 1V,2V,5V output options from the SQ-1 to modulate the frequency in a 1,2 and 5-octave ranges. The product has been integrated into a Euro Rack synthesiser format. This was integrated using 2 ADSR modules attenuating the volume of a voltage controlled low pass filter and amplifier modules. The product can output different waveforms which are Sine waves, square waves, triangle waves and parabola waves. The product uses multiple button inputs to select the different Wave Form options. The product can combine waveforms in runtime with harmonics to simulate plucked notes with inputs to add and subtract even and odd harmonics seperately. The product has an input to enable vibrato functionality that modulates the frequency up and down a semitone with an interval set so that it modulates at from each extreme every 1/3 of a second. The product has an additional oscillating voltage output using PWM, the frequency of which is determined by the CV-B output from the Korg SQ-1. This was integrated into the euro rack format by adding additional ADSR and amplifier modules. The Secondary has a secondary waveform selection allowing for independent control of wave form for each output with an additional input that switches the buttons to alter the secondary output waveform and vibrato.

# Appraisal

A lot of effort was wasted during this project from not fully understanding how PWM with the Pico worked but once that had a better understanding of the methods was acquired the project moved along more smoothly. There was a lot of time and effort put towards the wave table solutions and a lot less time on the runtime calculation. Someone were to attempt this project then more time should be spent looking into those solutions and implementing them into the final product.

A lot of effort was wasted was with not fully understanding the full scope of the project due to poor planning so that at the start some efforts were made that didn’t end up move the project along like when Ross Grosset’s tool for turning .wav files into wave tables was used to create a series of note wave tables from samples from the internet but this ended up being worthless to the project after the python program was developed and clock divider solution was implemented. While implementing multiple buttons, a lot of time was spent trying to implement a solution that was replaced using Jake Rosoman’s solution due to of a lack of knowledge of the C programming language, more specifically not knowing that pointers to functions could be members of structs and not understanding some of the uses of pointer that Jake Rosoman used.

It would also have also been beneficial to design the circuit near the start that reflects the outcomes of the project so that a PCB and front panel could have been created.

One thing that was really helpful was using an amplitude field while creating the python script since it made it a lot easier to visualise the wave form being generated.  A lot more effort should have gone into implementing the runtime waveforms because they had a lot more potential for generating equally high fidelity low and high frequency waveforms due to its ability to increase the number of accurate levels that are used at runtime.

# Summary and Conclusions

In summary this report shows how pulse width modulation was in the development of an Oscillating voltage generated by the Raspberry Pi Pico. It contains reference to the methods that were implemented from the pico hardware library documentation. This report also documents the specification of the product and shows what design choices were made to fufil the requirements. The report contains a full walk through of all the steps that lead to the final products implementation from generating the first controlled frequency to then generating different wave shapes using the wave table method to read values from an array that specifies a series of level values that produce a specific wave form. Then it details the development of runtime solutions that calculate the level of the pwm output at runtime. The report details the integration of the prototype with the Korg SQ-1 sequencer and the development of 2V and 5V solutions. Then the development of vibrato functionality was documented aswell as the integration of an additional PWM output.

The report details the results of the tests that were carried out during the implementation.

# Future Work

Although the project meets all the requirements set out for this project, a lot more features could be added. One of which could be implementing an additional ADC component that is controlled by a potentiometer to alter the rate of change of the vibrato. This ADC could also be used to change the sample rate of the output which could produce a lower fidelity effect that could be useful in certain musical contexts. The runtime methods that were researched in this project could be integrated into the final product and the additional ADC could alter runtime wave forms. The product could also use the Additional ADC input modulate the level of harmonics, changing the representation of the harmonics in the output, this would also require an additional input to change the wave form buttons to select each harmonic then the potentiometer could change the level of the selected harmonic.

To make the product commercially viable a PCB would have to be created for the circuitry. Once a PCB is integrated then it could be integrated as a eurorack module by creating a front panel design, with room for the buttons and mono jack outputs. It would also need the buttons to be changed for larger ones that would be able to stick out of the front panel. It would also require attaching the PCB to the front panel in a way that the PCB conforms to the front panel. A method for adapting the euro rack power connector so that it could power the Pico would also be required this would be done by taking the 5V wire from the euro rack power cable and connecting it to pin 39 (VSYS) on the Pico and connecting the ground wire from the cable to pin 38 (GND). It would probably mean changing the circuit to use another ground pin for the CV A input as well. The IOVDD voltage could be increased which could allow for the higher notes to be represented more clearly.

## Acknowledgments

I would like to acknowledge Ross Grosset’s pico-pwm-audio project which supplied some of the foundational code that was used to develop the product. I would also like to acknowledge Jake Rosoman’s pico-button and pico-gpio-interrupt project which supplied the code in button.c which allowed for the integration of multiple buttons into the product.

# References

https://github.com/jkroso/pico-button.c

https://github.com/jkroso/pico-gpio-interrupt.c

https://www.i-programmer.info/programming/hardware/14849-the-pico-in-c-basic-pwm.html

# https://datasheets.raspberrypi.com/pico/raspberry-pi-pico-c-sdk.pdf

# Appendices